Quality    management manual

Vision: Provide World Class Quality

Mission: Collaborate and align the entire project delivery team to place Quality Assurance over Quality Control by proactively educating the PDT while utilizing a P-D-S-A cycle that measures results and holds ourselves accountable to achieving the CoS and Vision.

Contents

INTRODUCTION	3

R&M’S QUALITY AT THE SOURCE  PROGRAM OVERVIEW	4

1.1	Condistions of Satisfaction					4

1.2	General Content – Quality Management Manual & Scope Specific QMM	4

1.3	Roles and Responsibilities					4

PRE-CONSTRUCTION QUALITY ACTIVITIES	7

2.1	Design Peer Reviews						7

2.2	Constructability Reviews					7

2.3	Review of Plans and Specifications				7

2.4	Documenting Existing Conditions					7

2.5	Trade Partner Selection					8

2.6	Samples, Submittals and Procurement				8

2.7	Factory/Shop Inspections					8

2.8	Material Receiving at the Jobsite					8

2.9	Mock-Ups							8

2.10	Envelope Moisture/Water Intrusion Testing				8

CONSTRUCTION QUALITY ACTIVITIES	10

3.1	Construction Goals						10

3.2	Quality Control Adminstration					10

3.3	Inspection and Testing Plan					11

3.4	Inspection Checklists						11

3.5	Non-conformance Tracking Procedure				11

3.6	Quality at the Source (QaS) Education				12

3.6.1        Definable Features of Work (DFW)				12

3.6.2        Plan-Do-Study-Adjust Cycle					12

3.6.3        Pre-Construction Meeting					12

3.6.4        Quality at the Source Meeting					12

3.6.5        First Run Study						13

3.6.6        Quality Report Out Meetings & Inspections				13

3.7	One Hundred Percent Material Verification				14

3.8	Digital Photography Requirements				14

3.9	Pre-cover-up and Pre-closure Inspections and Photographs		14

3.10	Construction Scheduling					14

3.11	Inspections During Construction					14

3.12	Moisture Control Plan and Procedure	                		 	14

TURNOVER/POST-CONSTRUCTION ACTIVITIES	16

4.1	Warranty	                                                                              		 	16

4.2	Warranty Call-backs During the Warranty Period			16

4.3	Warranty Call-backs After the Warranty Period			16

4.4	Turnover/Post-Construction Documentation	                   			16

# INTRODUCTION

“Quality is never an accident. Quality is the result of intelligent effort and a culture of continuous improvement that places quality assurance at the front of every process"

R&M understands that quality needs to be at the front of all processes and is critical to a successful project. We are committed to bringing quality to the front of all processes to minimize the unnecessary waste of re-work.  Therefore, by implementing a Quality Management Program (QMP), R&M plans to significantly reduce unnecessary waste.

The Program is referred to as Quality at the Source out of respect for the craftworkers who are putting the work in place.  Therefore, it is leadership’s obligation to assure the craftworkers (The Source) have all of the prerequisite education, experience and information they need to perform their scope of the project to achieve the Conditions of Satisfaction.

This manual has been specifically developed for The Wofford College Projects and is incorporated into all aspects of the project.

Consistent quality is the result of commitment and participation by every project member, this includes the owner, designers, trade partners, 3rd parties, AHJ’s, R&M and any other party with a vested interest in the project.

This Quality Management Manual (QMM) outlines R&M's Quality Assurance (QA) and Quality Control (QC) plan, standards and procedures that comprise R&M's Quality Management Program, hereinafter, the "Program." The purpose of this QMM is 3-fold:

To ensure world class quality standards are utilized to build the project.

To eliminate waste through continuous improvement while we deliver a best-in-class project.

To become a learning organization that provides guidelines to our project delivery team through Plan-Do-Study-Adjust (PDSA) cycles.

This program is a living document that will be assessed on regular intervals as needed by the Project.

# R&M’S QUALITY AT THE SOURCE  PROGRAM OVERVIEW

  - Conditions of Satisfaction

- Develop pride and ownership of quality

- Develop a collaborative project specific process

- Develop an efficient P-D-S-A process that eliminates redundant inspections

- Develop a process that educates, measures and holds each party accountable

- Design in quality and utilize First Run Studies to prevent repetitive quality issues

- Use quality as a selection criteria of trade partners

- Educate the entire PDT on the benefits of the Quality Management Program

- Reduce our carbon footprint by eliminating waste and sustainable practices

  - General Content – Scope Specific Quality Management Plan

A scope specific Quality Management Plan shall be prepared by all trade partners, with input and review from the PDT prior to commencing work.

All changes will be agreed to and incorporated into the plan.

Quality Management Plans shall incorporate key procedures and language from the contract documents in the following construction phases.

  - Pre-construction

  - Construction

  - Post-construction

  - Roles and Responsibilities

Each trade partner is responsible for developing, educating and implementing a Quality Management Plan including Quality at the Source (QaS) training to educate and inform relevant staff of the importance and need for this Program.

R&M is responsible for defining and monitoring compliance with this Quality Management Program on the Project.

Parties w/a vested interest are responsible to participate to the degree they are vested

R&M’s scope will be determined by the MIC (Manager In Charge) and the OM, examples are:

- Assist trade partners with development and implementation of the QMP

- Participate in identifying quality requirements

- Assess the need for additional procedures, tests and/or third-party oversight.

- Assure QaS training is conducted

- Oversee the software and process for analytics and documentation

- Assure participation as necessary from all vested members of the PDT

- Assist trade partners with the analytics to eliminate repetitive issues

- Monitor trade partners' daily QMP implementation

- Update and revise the QMP on a regular basis

- Facilitate Operational Learning Teams (OLT) developed to address specific issues

- Report on the performance of the Program when requested

- Conduct retrospectives with trade partners to learn and improve.

Trade partners scope will be in their QMP and will include:

- Prepare and submit a Quality Management Plan

- Develop QaS program to include check list and all contractual requirements

- Designate an accountable Site Quality Control Supervisor (SQCS)

- Utilize Procore for all quality issues tracking (Including Safety and Incomplete work)

- Direct coordination with all project team members

- Participate in OLTs as required

- Maintain document control and communication to the craftworkers

- Attend and participate in the weekly quality report out meetings

- Develop and report out on quality metrics

- Take action to prevent recurring items through the PDSA cycles

- Compliance with this program is condition precedent for payment

Owner/Designer/Others responsibilities*

- Review and provide comments on all Quality Management Plans, including lists

- Attend and participate in the Quality at the Source program and meetings

- Designate an authorized representative to manage quality

- Utilize Procore for all Quality issues tracking

- Input all known quality issues and sign off complete items on a timely basis

- Direct coordination and communication with all project team members

- Participate in OLTs as required

- Attend and participate in the weekly Quality meetings

- Communicate concerns to prevent recurring items through the PDSA cycles

Whether contractually obligated or not, it is in all parties best interest to achieve World Class Quality so participation in the QMP is critical to achieving the projects goals.  If at all possible, this Program should be incorporated into the contract(s).  If not agreed to a Comment & Clarification should be made in the contract with the owner.  “Robins & Morton holds quality at the top of the projects priorities, to this end, the attached QMP requires participation by all parties to achieve this goal.  All parties participation with this high performing plan is condition precedent to achieve World Class Quality”.

# PRE-CONSTRUCTION QUALITY ACTIVITIES

Pre-construction activities occur prior to commencing physical construction at the jobsite. These pre-construction activities frequently revolve around project budgets and decisions made at this stage  can significantly impact constructability and quality performance. R&M will coordinate its pre-construction services with Owners, Architects, Engineers, trade partners and other consultants.

The following paragraphs describe the pre-construction activities as they relate to the Quality.

  - Design Peer Reviews

R&M will collaborate with Owners to determine when peer reviews may improve the process. The purpose is to assess the adequacy, integration and completeness of the design documents, code compliance and conformance with manufacturer, supplier and vendor requirements including warranties. Possible peer review consultants may include but are not limited to: structural, MEP, acoustic, vibration, waterproofing/roofing, lighting, ADA or code compliance.

  - Constructability Reviews

R&M and trade partners will provide constructability reviews to be performed by Superintendents, PMs and Estimators collaboratively with Owners and project consultants. Early involvement by the project team during these reviews will reduce the likelihood of incorporating products or designs prone to installation or functional difficulties. Extra emphasis should be placed on reviews of high-risk, one-of-a-kind, or unusual design elements at both the DD and CD stages of design.

  - Review of Final Plans and Specifications

Before beginning construction R&M's superintendents and PMs will perform an in-depth review of the contract documents. This review will seek to identify conflicts, unclear or incomplete items, etc. A detailed list of comments and clarifications will be developed to openly communicate our understanding and interpretations of the design.

  - Documenting Existing Conditions

R&M will inspect and create a photographic and/or video record that documents existing conditions paying particular attention to preexisting physical defects. Items to be photographed include:

  - Existing structures within or adjacent to the construction limits, to remain

  - Adjacent buildings and structures outside the construction limits which may affected by construction activities

  - Adjacent roads, sidewalks, curbs, paving and drainage structures

  - Trees and existing vegetation

  - Above-ground utilities

  - Trade Partner Selection

Trade partner selection shall follow R&M's trade partner pre-qualification and selection criteria, which include safety, quality, capability and capacity evaluations.

  - Samples, Submittals and Procurement

R&M will utilize a comprehensive submittal log, to assure all submittals and samples are provided and approved.  The submittals will be reviewed for compliance and notes for QA will be added to the QA Checklists in each trade contractors’ QMP.

Submittal samples shall demonstrate the level of quality that is acceptable to the design team. When specified, This may be a mock-up outside of the actual area of construction or in other cases, it may be the first acceptable unit of production. These samples mock-ups will constitute the acceptable level of workmanship.

  - Factory/Shop Inspections

As part of the pre-construction planning process, the Project Team may elect to tour manufacturing assemblies or other projects to confirm the quality of a product and/or installation.  This will be documented and tracked on the OPS.

  - Material Receiving at the Jobsite

Trade partners' SQCS or designee will verify that 100% of the items delivered to the jobsite are in conformance with approved submittals and specifications upon receipt.

  - Mock-Ups

As part of the pre-construction planning process, R&M's PM will confirm the owner's contractual requirements for in-situ or freestanding mock-ups. The mock-up design, construction and review phases will be indicated on the Overall Project Schedule. In addition to these contractually defined mock-ups, R&M's PM will identify other high-risk construction assemblies as candidates for additional in-situ or freestanding mock-ups, such as:

  - Exterior skin

  - Roofing

  - Window and door penetrations

  - Pre-cast or GFRC panels

  - Drywall finish/texture

  - Wall/ceiling finishes

  - Floor finishes

  - Cabinets and millwork

  - Countertops and fixtures

  - Hardscape

  - For mock-up units, such as, bathrooms, headwalls, wet walls, and ceiling racks, R&M's Project Manager will determine the exact number and type of mock-ups to be constructed. Mock-ups will be utilized to establish constructability and appropriate levels of quality.

  - Envelope Moisture/Water Intrusion Testing

As part of the pre-construction planning process, R&M's PM will confirm the owner's contractual requirements for water testing. In addition to these contractually defined water tests, R&M will follow the Moisture Control Policy which may identify other high-risk construction assemblies as candidates for additional water tests that may be performed by a third-party consultant, such as:

  - Exterior skin

  - Curtainwall or storefront systems

  - Windows

  - Skylights

  - Exterior doors, thresholds and store fronts

  - Balcony and deck transitions to the building wall

  - Planters above occupied spaces

  - Roofing Systems

  - Shower pans

  - Fountains

# CONSTRUCTION QUALITY ACTIVITIES

3.1      Construction Goals

- To have zero punch list items at the time of substantial completion

- Education of all craft workers on project specifications and requirements

- Leadership development of the last planners

- To correct defects within seven (7) calendar days of notice.

- Use the data from Procore for continuous improvement training

- Maintain zero past due items

- Maintain zero average days late for response time of punch list

It is the intent of this program to obtain 100% buy-in from the entire project delivery team. Everyone must participate in order for the program to succeed.

3.2    Quality Control Administration

Procore will be used for all quality documentation, including pictures. See Procore Best Practice for details.

Implementation

- The project team consultants and 3rd party inspectors will be required to input all identified deficiencies into Procore. Project team members will have access to this data base and be required to follow the specific QMP.

- All deficient items will be input into Procore as a “Punch List” item. Any item not input as a “Punch List” item will not be recognized nor addressed as a defect.

Workmanship Inspections

Project team members should inspect all items that will be covered-up by a follow-on operation. Any issues or concerns requiring action must be entered into Procore as a “Punch List” item. This includes any items or issues in designer or consultant’s reports.  If items are not entered in Procore, they will not be corrected.

Documentation

Documentation will be archived in Procore.

Inspection and Testing Log

As inspections and tests are performed they must be entered in Procore. Logs may include items like:

  - Specification section

  - Witnessed by Owner, Architect, Engineer or Commissioning Agent

  - Description of the test performed

  - Location of the test

  - Who performed the test

  - Date the test was conducted

  - Any remarks to appropriate parties

Defects found during inspection and testing must be added to Punch List or they will not be corrected.

Punchlist & Final Inspection

After the installation or near the completion of all Definable Features of Work, SQCS or project superintendent will conduct an inspection of the work and input defective items in Procore’s “Punch List”. Punchlist items are required to be completed within 7 days or addressed in writing. The author of the item will make follow-up inspections within 7 days or when notified to validate correction and close the item in Procore.

3.3    Inspection and Testing Plan

As a part of the Quality Management Program, R&M will verify each trade partner’s inspection and testing process. This process will delineate all inspections and tests required by the contract documents. Testing and inspections will be coordinated and shown on the Construction Near Term Schedule (CNTS). Major inspections and certifications will be shown on the Overall Project Schedule (OPS).

It is best practice to involve manufacturer's representatives in inspections and approvals for high risk building elements such as: Roofing systems, Window systems, Other elements of the building envelope Specialized and/or proprietary installations such as equipment, flooring systems, etc.

  - Inspection Checklists

Inspection Checklists will be developed by their respective trade contractor responsible for each Definable Feature of Work. Upon mobilization these lists will be reviewed with craft workers in the QaS meetings. The inspection checklist should include all pertinent steps required to perform the DFW.

All inspection templates and completed inspection checklists will be input into Procore.

  - Non-conformance Tracking Procedure

A non-conformance item, also known as a deficiency, or defect is work that doesn’t meet project requirements. A defect can be either material-related, workmanship related, design-related or a combination thereof.  It is expected that all non-conforming items will be corrected within 7 days of being identified in Procore.

Non-conforming items will be documented on Procore’s “Punch List”.

  - Quality at the Source (QaS) Education

3.6.1 Definable Features of Work (DFW)

DFW refers to an individual scope of work as defined by each specification section (Reference Construction Specifications Institute)

Each DFW will have separate QA/QC requirements. The inspections (QC) will be performed by the contractually responsible party, The education and information will be provided by the contractor contracted to perform the DFW.  The details will be listed in the respective Quality Management Plans.

3.6.2 Plan-Do-Study-Adjust Cycle

- Quality Management Plan = “Plan”

- Educate and Install = “Do”

- First Run Studies and Weekly Quality Meetings = “Study”

- Operational Learning Teams (OLT) and Counter Measures = “Adjust”

3.6.3 Pre-Construction Meeting

A pre-construction meeting will be facilitated by R&M on each significant construction activity or Definable Feature of Work (DFW.) The following will be in attendance:

• Trade partners

• Owner's representative(s)

• Design professional(s)

• Third party consultants/inspectors

• Manufacturer's/Vendor representatives

• Robins & Morton Project Manager*

• Robins & Morton Superintendent

The goal of this meeting is to align contractors/designers/& owner’s expectations. This will include focusing on the concept of preventing deficiencies (quality assurance).

3.6.4 Quality at the Source Meeting

A Quality at the source meeting will be conducted by trade partners (last planners) responsible for each DFW. It is important that a QaS meeting is performed for each craft worker on the jobsite. The following will be in attendance:

• Trade partners

• Supervisor (last planner)*

• Craft workers

• Owner's representative(s)

• Design professional(s)

• Third party consultants/inspectors

• Manufacturer's/Vendor representatives

• Robins & Morton superintendent

The goal of this meeting is to communicate the quality expectations and design intent for each DFW to the craft workers.

These meetings should be video taped.

3.6.5 First Run Study

A First Run Study is a field inspection that should be performed immediately following the start of construction for a DFW.

This First Run Study will be facilitated by R&M and led by trade partner at the scheduled time (typically between 1 week and 1 month after the DFW starts). All First Run Studies should be included in the schedule and updated along with changes to the schedule. This meeting will be attended by the following individuals:

• Trade Partners*

• Robins & Morton superintendent

• Owner's representative

• Design professional(s)

• Third party consultants

• Manufacturer's representatives

• Predecessor and successor trades

The goal of the First Run Study is to ensure compliance with project requirements and to confirm all predetermined expectations and intentions have been met. This study should be the forum to capture all deficiencies at the front end of the DFW.  If deficiencies are found, a second QaS meeting may be necessary to ensure quality going forward.

First run studies should validate the check list established in the pre-construction & QaS meetings.

• Verify that manufacturer's installation instructions are being followed

• Check for use of defective or damaged materials

• Check new work for compliance with construction documents

• Check safety compliance

• Establish the acceptable level of workmanship

• Verify that all materials are in compliance with construction documents

• Review and approve testing and inspection results

• Check for omissions and resolve any differences of interpretation

3.6.6 Quality Report Out Meetings & Inspections

The Quality Report Out Meeting is a weekly gathering of all trades to report out quality updates and discuss any open quality items. See attached agenda.

  - One Hundred Percent Material Verification

For all applicable DFWs, trade partners shall verify that materials received at the jobsite are in compliance with contract documents.

During Follow-up inspections and First Run Studies - R&M & trade partners will continue to verify that approved materials are being installed.

  - Digital Photography Requirements

Photos are an integral part of the QMP. Digital cameras and iPads will be used to document work in Procore. Although it is not practical to photograph every inspected item, R&M’s intent is to provide representative photographic documentation for each DFW and project phase.

  - Pre-cover-up and Pre-closure Inspections and Photographs

R&M will inspect all items that will be covered-up by a following operation. R&M will inspect all items before covering up. Photographs or videos of all work to be covered-up will be accomplished through Procore, Multi-Vista and/or digital photography.

This procedure applies to floors, ceilings, walls and below ground work, such as, footings, stem walls, basement walls, and below ground utilities.

  - Construction Scheduling

All parties will review and confirm all quality requirements are documented and acceptable per the OPS.  After review and comments are received and incorporated from all parties, the OPS will be incorporated into each subcontract.

Consideration must be given to the three distinct construction periods when evaluating the potential for building and vulnerable construction materials being exposed to high moisture conditions.

These include,

- “Exposed Phase": This includes the period when all building materials are exposed to precipitation, which typically includes framing and placing of concrete.

- "Partially Enclosed Phase”: This includes the period after the roof deck is installed but the building is not fully weathered in. Typically, many vulnerable materials are installed during this phase of construction.

- "Enclosed Phase": This includes the phase when the building envelope and the roofing system are complete and the interior finishes are being installed.

  - Inspections During Construction

R&M will inspect the project daily and document any deficiencies in Procore, including digital pictures when relevant. Owner's, designers, consultants and other relevant PT members will inspect the project continually when on site and ALL defects, concerns or other issues must be documented via Procore’s “Punch List” to be recognized. The inspection frequency will be dependent on project conditions but at a minimum should be completed every week.

  - Moisture Control Plan and Procedure

R&M Moisture Control Policy will be utilized from preconstruction through close out for both QA/QC.

Owner’s participation with this plan should be coordinated and scheduled to avoid conflicts and confusion.

# TURNOVER/POST-CONSTRUCTION ACTIVITIES

- Warranty

- R&M provides a standard 1-year warranty program, with a single point of contact.

- R&M's warranty program will log warranty call-back requests, differentiating between maintenance and warranty items and ensuring prompt response. Coordinate with the Owner’s Representative with regards to the warranty program.

- If requested, a final warranty/1 Year inspection will be conducted 11 months after substantial completion if requested.

- All warranty issues will be tracked through closure.

- Warranty Call-backs During the Warranty Period

- R&M will endeavor to satisfy the warranty and post-construction issues of the project during the contractual warranty period in accordance with the specific terms and conditions of the construction contract.

- The turnover documents will contain a Subcontractor / Vendor Directory that lists the name, contact person and telephone number of each subcontractor and vendor.

- R&M as well as the MEPF subcontractors should be notified immediately if major problems occur with systems that affect or impact the operation of the facility.

- R&M will respond to a warranty call-back within 72 hours, 5 days per week, and requires similar responsiveness on the part of its trade partners.

- It is R&M's goal to resolve each warranty call within 30 days after receiving a warranty call-back request and requires a similar commitment from all trade partners.

- Warranty Call-backs After the Warranty Period

- Once the warranty period has expired, potentially responsible trade partners and suppliers may no longer be under a contractual obligation to respond. It nevertheless is R&M's goal to resolve such complaints if it is commercially feasible.

- Turnover/Post-Construction Documentation

Quality records will be submitted in an indexed, retrievable fashion. These records will become part of the project's permanent records and will be filed in such a fashion required by DHEC and/or Owner’s Representative.